Technical Documentation

# MQTT Virtual Device Dashboard
MQTTVirtualDeviceDashboard.jsx

Complete implementation reference for all features simulating a STLA M6 connected vehicle TBOX

Component

MQTTVirtualDeviceDashboard.jsx

Route

/mqtt-virtual-device

Framework

React 18 + Framer Motion

Doc Date

15 April 2026

Spec Version

SB Interface V5.0

Source Rev

Rev 1.1 — 15-Apr-2026

### Source Reference Documents

MQTT_Virual_Device_3012 — Rev 1.1 — 15-Apr-2026

TE 01 _ STLA M6 Connectivity South Bound Interface Specification V5.0

Jeep_Proto/*.proto — stla.cvip.jeep.*

certificates: ca.crt, client.crt, client.key

## Table of Contents

1

Executive Summary

2

Source Reference Files

2.1

MQTT Virtual Device Node.js Scripts

2.2

STLA M6 SB Interface Spec V5.0 Sections Used

3

Section 1 — Device Identity Configuration

4

Section 2 — MQTT Connection Panel

5

Section 3 — Publish Panel (All 6 Tabs)

5.1

Vehicle Telemetry — 5 CAN Signals

5.2

Location Telemetry + GPS Map

5.3

Events — 10 Event Types

5.4

Alerts — 10 Alert Types

5.5

Trips — Start / Current / End

5.6

Device Join (deviceJoined)

6

Section 4 — Command Inbox (8 Command Types)

7

Section 5 — Live Telemetry Cards

8

Section 6 — Message Log Panel

9

Automation — Simulate Auto Drive

10

MQTT Topic Reference (Complete)

11

State Management Architecture

12

Animation Spring Configurations

13

Visual Design System

14

What Is NOT Implemented (Scope)

15

How to Use the Dashboard

## 1 Executive Summary

The MQTTVirtualDeviceDashboard.jsx is a fully self-contained React 18 component that   simulates a connected vehicle TBOX (Telematics Box / Dongle) device communicating   over the MQTT protocol with protobuf-encoded messages over the CVIP (Connected Vehicle India Platform).   It is embedded inside the Rule Engine Dashboard at /mqtt-virtual-device.

⚡ Key Feature

No real MQTT broker is required.

All payloads are mock-simulated as structured JSON   matching the actual protobuf schema from the source Node.js scripts. The component can be used   entirely offline for diagnostic and testing purposes.

#### Dashboard Capabilities

- Configure full device identity: VIN, IMEI, TBOX Serial, Broker URL, Protocol Version

- Simulate MQTT connect / disconnect lifecycle with visual connection state machine

- Publish 6 categories of MQTT messages: Vehicle Telemetry, Location, Events, Alerts, Trips, Device Join

- Simulate 8 types of incoming server commands and send back command responses with correlationId

- View 6 live telemetry gauges with animated spring-count transitions and trend arrows

- Browse a scrollable, filterable message log with JSON + Hex payload expansion

- Auto Drive mode — continuous GPS drift + telemetry stream every 3 seconds

- Export full log as JSON for offline analysis

#### Technology Stack

| Library | Version | Usage |
| --- | --- | --- |
| react | 18.x | Component framework; useReducer state management |
| framer-motion | ^11.18.2 | All physics-based spring animations (replaces "Antigravity" engine) |
| uuid | ^13.0.0 | UUID v4 generation for messageId, correlationId, logId |
| Node.js mqtt | 5.8.0 | Source reference (not bundled); all MQTT logic is simulated |
| Node.js protobufjs | ^8.0.0 | Source reference (not bundled); mock hex payloads generated |

## 2 Source Reference Files Read

### 2.1 MQTT Virtual Device Node.js Scripts — Rev 1.1 (15-Apr-2026)

| Directory | Script File | Implemented Feature | In Dashboard |
| --- | --- | --- | --- |
| deviceJoin/ | deviceJoin.js | Device handshake / registration / topic setup | ✅ |
| vehicleTelemetry/ | vehicleTelemetryFuelLevel.js | CAN Fuel Level — identifier 854, PDU 0x64... | ✅ |
| vehicleTelemetry/ | engineSpeed.js | CAN Engine Speed (RPM) | ✅ |
| vehicleTelemetry/ | engineWaterTemp.js | CAN Engine Water Temperature (°C) | ✅ |
| vehicleTelemetry/ | vechileTelemetryBatteryVoltage.js | CAN Battery Voltage (V) | ✅ |
| vehicleTelemetry/ | odoMeter.js | CAN Odometer (km) | ✅ |
| locationTelemetry/ | locationTelemetry.js | GPS lat, long, alt, course, signal quality | ✅ |
| events/ | start_event.js | Ignition START event | ✅ |
| events/ | stop_event.js | Ignition STOP event | ✅ |
| events/ | remoteBlinkerOnOffCommand.js | Remote blinker ON / OFF events | ✅ |
| events/ | remoteDoorLockUnlockCommand.js | Door lock / unlock events | ✅ |
| events/ | remoteHonkEventStatus.js | Remote honk event status | ✅ |
| events/ | tboxInFullSleepStatus.js | TBOX full sleep entry event | ✅ |
| events/ | vehicleMobilizationStatus.js | Vehicle mobilization status | ✅ |
| alerts/ | speedAlertWithValue.js | Speed alert with vehicleSpeed value + GPS | ✅ |
| alerts/ | harshAccelerationAlert.js | Harsh acceleration alert | ✅ |
| alerts/ | harshBreakingALert.js | Harsh braking alert | ✅ |
| alerts/ | sharpTurnsAlert.js | Sharp turns alert | ✅ |
| alerts/ | engineIdlingAlert.js | Engine idling alert | ✅ |
| alerts/ | parkingDisturbanceAlert.js | Parking disturbance alert | ✅ |
| alerts/ | towAwayAlert.js | Tow-away alert | ✅ |
| alerts/ | dongleStatusAlert.js | Dongle health / status alert | ✅ |
| alerts/ | remoteImmobilizationAlert.js | Remote immobilization alert | ✅ |
| alerts/ | remoteMobilizationAlert.js | Remote mobilization alert | ✅ |
| trips/ | tripStart.js | Trip start lifecycle + start GPS | ✅ |
| trips/ | tripCurrent.js | Trip progress / current data | ✅ |
| trips/ | tripEnd.js | Trip end + total distance / duration | ✅ |
| command/ | remotedoorlockcommand.js | Door lock command response | ✅ |
| command/ | remoteDoorUnLockCommandResponse.js | Door unlock command response | ✅ |
| command/ | remoteblinkeroncommand.js | Blinker ON command response | ✅ |
| command/ | remoteblinkeroffcommand.js | Blinker OFF command response | ✅ |
| command/ | remotehonkcommand.js | Honk command response | ✅ |

### 2.2 STLA M6 SB Interface Specification V5.0 Sections Used

| Section | Topic | API Interface ID | Implemented |
| --- | --- | --- | --- |
| §6.2 | MQTT Protocol — QoS 1, MQTT v3.1.1, TLS 1.2 | — | ✅ |
| §6.2.1 | Connection flags: Clean Session=false, ClientId=VIN/IMEI | — | ✅ |
| §6.2.4 | Keep Alive: 30s (configurable) | — | ✅ Documented |
| §6.2.5 | LWT Topic: /dongle/{VIN}/MQTTPROTOBUF/deviceJoined | — | ✅ Documented |
| §7.3.4.1 | deviceJoined / deviceJoinedRsp message format | SB-DONGLE_ACTIVATION-007 | ✅ |
| §7.3.4.1.3 | Regex validation rules for deviceJoined parameters | — | ✅ |
| §8.1 | Topic Nomenclature: /dongle/{VIN}/MQTTPROTOBUF/{type} | — | ✅ All 13 topics |
| §9 | Dongle Commands (command / commandResponse topics) | SB-DONGLE_COMMANDS-001 | ✅ |
| §9.2 | Fetch Device Logs command | SB-DONGLE_COMMANDS-003 | ✅ |
| §9.4 | Dongle State Update command | SB-DONGLE_COMMANDS-004 | ✅ Simulated |
| §10.7 | Remote Blinker ON Command | SB-REMOTE_COMMANDS-001 | ✅ |
| §10.8 | Remote Blinker OFF Command | SB-REMOTE_COMMANDS-002 | ✅ |
| §10.9 | Remote Door Lock Command | SB-REMOTE_COMMANDS-003 | ✅ |
| §10.10 | Remote Door Unlock Command | SB-REMOTE_COMMANDS-004 | ✅ |
| §10.11 | Remote Honk Command | SB-REMOTE_COMMANDS-005 | ✅ |
| §10.12 | Remote command failure codes (26 failure codes) | SB-REMOTE_COMMANDS-006 | ✅ Documented |
| §11 | FOTA — fotaCommand / fotaCommandResponse topics | SB-FOTA_COMMANDS-001 | ✅ Simulated |
| §12.2 | Vehicle Telemetry stream (CAN data) | — | ✅ |
| §12.3 | Location Telemetry stream (GPS) | — | ✅ |
| §13.1 | Vehicle Alerts (10 types) | — | ✅ |
| §13.2 | Vehicle Events (10 types) | — | ✅ |
| §13.11 | Trip Start payload | SB-TRIP_TELEMETRY-006 | ✅ |
| §13.12 | Trip Current/Progress payload | SB-TRIP_TELEMETRY-006 | ✅ |
| §13.13 | Trip End payload | SB-TRIP_TELEMETRY-006 | ✅ |

## 3 Section 1 — Device Identity Configuration

Source: .env from MQTT Virtual Device Rev 1.1 + deviceJoin.js lines 19–28

### 3.1 Configurable Fields

| Field | Default Value | .env Variable | Spec Reference |
| --- | --- | --- | --- |
| VIN | T123ZTZT867657777 | VEHICLE_ID | §7.3.4.1 vehicleId, regex: ^[a-zA-Z0-9]{17,20}$ |
| IMEI | 356741396798769 | IMEI | §7.2.1 15-digit IMEI, TAC="35674136" |
| TBOX Serial | T123ZTZT1396798769 | TBOX_SERIAL | §7.3.4.1 tboxSerialNum |
| Client ID | T123ZTZT867657777 | CLIENT_ID | §6.2.1 — VIN after provisioning |
| Message ID | UUID v4 | MESSAGE_ID | §8 — "UUIDs based on current timestamp" |
| Correlation ID | UUID v4 | CORRELATION_ID | §4 Glossary — "message ID of corresponding command" |
| Broker URL | cvipiot-preprod.fca-india.com:18883 | BROKER_URL | §6.2.6 TLS 1.2 Preprod |
| Protocol Version | 2.0.0 | — | §8 "version: 2.0.0" — all payloads |

### 3.2 Broker Options (Dropdown)

| Label | Full URL | Port |
| --- | --- | --- |
| FCA India Preprod | mqtts://cvipiot-preprod.fca-india.com | 18883 |
| Citroen Preprod | mqtts://lb2.cvip-preprod.citroen.in | 48883 |

### 3.3 Action Buttons

- ↻ Generate UUIDs — Refreshes both messageId and correlationId with new UUID v4 values

- 💾 Save Config — Persists draft form values to global React state + fires toast notification

## 4 Section 2 — MQTT Connection Panel

Source: deviceJoin.js mqtt.connect() options + Spec §6.2

### 4.1 Connection State Machine

| State | Color | Dot Animation | Trigger |
| --- | --- | --- | --- |
| DISCONNECTED | Gray #52525b | Static | Initial state / Disconnect button |
| CONNECTING | Amber #f59e0b | Scale pulse loop, 0.9s | Click ▶ CONNECT |
| CONNECTED | Green #10b981 | Slow pulse loop, 2.0s | ~800ms after CONNECTING |
| ERROR | Red #f43f5e | Rapid shake oscillation | Simulated network failure |

### 4.2 Live Connection Stats

| Stat | Display | Update Trigger |
| --- | --- | --- |
| Published | Integer counter | Every successful onPublish() |
| Received | Integer counter | Every simulated SUB message |
| Uptime | HH:MM:SS format | setInterval 1000ms while CONNECTED |
| Last Pub | HH:MM:SS.ms | Most recent publish timestamp |

### 4.3 MQTT Parameters (from source + Spec)

clientId:        CLIENT_ID (VIN/IMEI — per §6.2.1) protocolVersion: 4 (MQTTv3.1.1 — per §6.2) clean:           false (persist session — per §6.2.1 "Clean Session = 0") keepalive:       30 seconds (§6.2.4: 10 minutes in spec, 30s simulated) QoS:             1 (at least once — per §6.2.2) LWT topic:       /dongle/{VIN}/MQTTPROTOBUF/deviceJoined (§6.2.5) TLS version:     1.2 (§6.2.6 Preprod environment)

## 5 Section 3 — Publish Panel (All 6 Tabs)

### 5.1 Vehicle Telemetry Tab

Source: vehicleTelemetry/*.js — Spec §12.2 — Proto: stla.cvip.jeep.CANDataMessage

Published Topic:/dongle/{VIN}/MQTTPROTOBUF/vehicleTelemetry

#### CAN Signal Types (Dropdown)

| Signal | CAN Identifier | Unit | PDU Default | Source File |
| --- | --- | --- | --- | --- |
| Fuel Level | 854 | % | 0x6400000000000000 | vehicleTelemetryFuelLevel.js |
| Engine Speed | 100 | RPM | Configurable decimal input | engineSpeed.js |
| Engine Water Temp | 100 | °C | Configurable decimal input | engineWaterTemp.js |
| Battery Voltage | 100 | V | Configurable decimal input | vechileTelemetryBatteryVoltage.js |
| Odometer | 100 | km | Configurable integer input | odoMeter.js |

#### Full Payload Structure (from vehicleTelemetryFuelLevel.js)

{   "messageId": "dd52bab1-c64a-431b-a945-1602f26e9f4e",   "eTboxApplicationState": "customer",   "tboxEsimState": "normal_sim",   "tboxOperatingState": "normal",   "version": "2.0.0",   "timeStamp": { "seconds": 1750000000, "nanos": 0 },   "canDataPayload": {     "canData": {       "canDataPacket": [{         "identifier": 854,         "dlc": 8,         "cData": [{           "pdu": "0x6400000000000000",           "timestamp": { "seconds": 1750000000, "nanos": 0 }         }]       }]     }   } }

#### Additional Configurable Fields

| Field | Options | Proto Enum |
| --- | --- | --- |
| App State | customer / provisioned / authorized_customer / provisioned_authorized | eTboxApplicationState |
| eSIM State | normal_sim / no_sim / roaming_sim | tboxEsimState |
| Operating State | normal / sleep / full_sleep / boot | tboxOperatingState |
| Interval (sec) | 1–3600 (number) | Auto-repeat timer |
| Auto Toggle | ⟳ Auto / ⏹ Stop | setInterval / clearInterval |

### 5.2 Location Telemetry Tab

Source: locationTelemetry/locationTelemetry.js — Spec §12.3 — Proto: stla.cvip.jeep.LocationTelemetryMessage

Published Topic:/dongle/{VIN}/MQTTPROTOBUF/locationTelemetry

| Field | Type | Default (Chennai) | Spec Field |
| --- | --- | --- | --- |
| GPS Latitude | double | 13.084534 | gpsLat |
| GPS Longitude | double | 80.270718 | gpsLong |
| GPS Altitude | float | 20.0 m | gpsAlt |
| GPS Course Angle | float | 180.0° | gpsCourseAngle |
| GPS Signal Quality | float | 97.0 | gpsSignalQuality |
| GPS Fixed Status | int32 | 1 (Fixed) | gpsFixedStatus |
| Speed | float | 25 km/h | speed |
| Signal Strength | int32 | 30 | signalStrength |

🗺 Mini GPS Map

A 400×180px dark grid map renders an animated amber pin that tracks lat/long inputs in real-time using   Framer Motion

useSpring

(stiffness: 120, damping: 18). The pin shows current coordinates   as text, and moves smoothly when values change.

### 5.3 Events Tab

Source: events/*.js — Spec §13.2 — Proto: stla.cvip.jeep.EventMessage

Published Topic:/dongle/{VIN}/MQTTPROTOBUF/events

| Event Type | Source Script | Spec Reference |
| --- | --- | --- |
| IgnitionStatus: START | start_event.js | §13.2 IgnitionPropertyType.START |
| IgnitionStatus: STOP | stop_event.js | §13.2 IgnitionPropertyType.STOP |
| RemoteBlinkerOn | remoteBlinkerOnOffCommand.js | §10.7 RemoteBlinkerOnCommand |
| RemoteBlinkerOff | remoteBlinkerOnOffCommand.js | §10.8 RemoteBlinkerOffCommand |
| RemoteDoorLock | remoteDoorLockUnlockCommand.js | §10.9 RemoteDoorLockCommand |
| RemoteDoorUnlock | remoteDoorLockUnlockCommand.js | §10.10 RemoteDoorUnLockCommand |
| RemoteHonkStatus | remoteHonkEventStatus.js | §10.11 RemoteHonkCommand |
| VehicleMobilizationStatus | vehicleMobilizationStatus.js | §13.2 |
| TBoxInFullSleep | tboxInFullSleepStatus.js | §11.1.4.1 Full Sleep |
| VehicleMobilization | vehicleMobilizationStatus.js | §13.2 |

### 5.4 Alerts Tab

Source: alerts/*.js — Spec §13.1 — Proto: stla.cvip.jeep.AlertsMessage

Published Topic:/dongle/{VIN}/MQTTPROTOBUF/alerts

| Alert Type | Source Script | Extra Fields |
| --- | --- | --- |
| SpeedAlert | speedAlertWithValue.js | vehicleSpeed (km/h) + GPS lat/long |
| HarshAcceleration | harshAccelerationAlert.js | alertState, GPS |
| HarshBraking | harshBreakingALert.js | alertState, GPS |
| SharpTurns | sharpTurnsAlert.js | alertState |
| EngineIdling | engineIdlingAlert.js | alertState |
| ParkingDisturbance | parkingDisturbanceAlert.js | alertState |
| TowAway | towAwayAlert.js | alertState, isLive |
| DongleStatus | dongleStatusAlert.js | alertState |
| RemoteImmobilization | remoteImmobilizationAlert.js | alertState |
| RemoteMobilization | remoteMobilizationAlert.js | alertState |

#### Alert State Options (from proto enum)

alsAlert

(active alarm)  |

alsNormal

(cleared)  |

alsUnknown

(indeterminate)

### 5.5 Trips Tab

Source: trips/*.js — Spec §13.11–13.13 — Proto: stla.cvip.jeep.TripMessage

| Sub-Tab | Published Topic | Key Fields |
| --- | --- | --- |
| Trip Start | /dongle/{VIN}/MQTTPROTOBUF/tripStart | tripId (date-based int64), startTime, startDate, gpsLat, gpsLong, gpsAlt, gpsCourseAngle, gpsFixedStatus |
| Trip Current | /dongle/{VIN}/MQTTPROTOBUF/tripCurrent | tripId, distanceSoFar (km), currentSpeed (km/h), currentTimeStamp |
| Trip End | /dongle/{VIN}/MQTTPROTOBUF/tripEnd | tripId, endGpsLat, endGpsLong, totalDistance (km), duration (minutes) |

Trip ID Generation

Auto-generated as

DDMMYY + "03"

encoded as int64. E.g., for 15-Apr-2026 →

150426

. Source:

tripStart.js

line ~20.

### 5.6 Device Join Tab

Source: deviceJoin/deviceJoin.js — Spec §7.3.4.1 — SB-DONGLE_ACTIVATION-007

Published Topic:/dongle/{VIN}/MQTTPROTOBUF/deviceJoined

Auto-Subscribe on Publish:/dongle/{VIN}/MQTTPROTOBUF/deviceJoinedRsp and /dongle/{VIN}/MQTTPROTOBUF/command

{   "vehicleId":          "T123ZTZT867657777",   "tboxSerialNum":      "T123ZTZT1396798769",   "imeiNo":             "356741396798769",   "protocolVersion":    "2.0.0",   "tboxOperatingState": "NORMAL",   "tboxApplicationState": "PROVISIONED",   "tboxeSimState":      "NORMAL_SIM",   "NAD_SW_Version":     "ND0.00.11",   "MCU_SW_Version":     "MD0.00.03" }

#### Regex Validation Rules — Spec §7.3.4.1.3

| Field | Regex / Allowed Values |
| --- | --- |
| vehicleId | ^[a-zA-Z0-9]{17,20}$ |
| tboxSerialNum | ^[a-zA-Z0-9]{13}$ |
| protocolVersion | ([0-9][.])([0-9][.])[0-9] |
| imeiNo | ^[0-9]{15}$ |
| tboxOperatingState | normal ¦ disconnected ¦ NORMAL ¦ DISCONNECTED |
| tboxApplicationState | factory ¦ provisioned ¦ authorized ¦ customer (any case) |
| tboxeSimState | normal_sim ¦ NORMAL_SIM |
| NAD_SW_Version | ^[a-zA-Z0-9]{3}.[a-zA-Z0-9]{2}.[a-zA-Z0-9]{2}$ |
| MCU_SW_Version | ^[a-zA-Z0-9]{3}.[a-zA-Z0-9]{2}.[a-zA-Z0-9]{2}$ |

## 6 Section 4 — Command Inbox (8 Command Types)

Source: command/*.js — Spec §9 (Dongle Commands) — SB-REMOTE_COMMANDS-001 to 006

Subscribe Topic:/dongle/{VIN}/MQTTPROTOBUF/command

Publish Response:/dongle/{VIN}/MQTTPROTOBUF/commandResponse

| Command | Source File | SB API ID | Subtype |
| --- | --- | --- | --- |
| Remote Door Lock | remotedoorlockcommand.js | SB-REMOTE_COMMANDS-003 | RemoteDoorLockCommand |
| Remote Door Unlock | remoteDoorUnLockCommandResponse.js | SB-REMOTE_COMMANDS-004 | RemoteDoorUnLockCommand |
| Remote Blinker ON | remoteblinkeroncommand.js | SB-REMOTE_COMMANDS-001 | RemoteBlinkerOnCommand |
| Remote Blinker OFF | remoteblinkeroffcommand.js | SB-REMOTE_COMMANDS-002 | RemoteBlinkerOffCommand |
| Remote Honk | remotehonkcommand.js | SB-REMOTE_COMMANDS-005 | RemoteHonkCommand |
| FOTA Command | fotaCommands/ | SB-FOTA_COMMANDS-001 | fotaCommand |
| OTA Command | — | FOTA extension | otaCommand |
| Fetch Device Log | fetchDeviceLog/ | SB-DONGLE_COMMANDS-003 | FetchLogsCommand |

#### Command Response Structure — from remotedoorlockcommand.js

{   "messageId": "dd52bab1-...",   "correlationId": "<extracted-from-command-messageId>",   "subtype": "RemoteDoorLockCommandResponse",   "tboxOperatingState": "normal",   "eTboxApplicationState": "customer",   "tboxEsimState": "normal_sim",   "returnCode": "succeeded",                  // succeeded | failed   "commandStatus": "success",                 // success | failure   "version": "2.0.0",   "timeStamp": { "seconds": ..., "nanos": ... } }

Spec §10.6 — Key Design Consideration

"If the command is received by Dongle

after 60 seconds

of command-initiated timestamp,   such commands need to be ignored by Dongle and need not to be processed." The command received-at   timestamp is visible in the dashboard for tester awareness.

#### Remote Command Failure Codes — Spec §10.12 (26 codes)

| Code | Name |
| --- | --- |
| 0 | invalidRemoteFailureCode |
| 1 | KeyIn_ACC |
| 2 | KeyIn_IGN |
| 3 | Engine_Running_or_Ready_ToDrive |
| 4 | LVBAT_LowVoltage_or_High_Voltage |
| 8 | Bounce |
| 9 | FotaInProgress |
| 16 | authentication_failed |
| 17 | authentication_timeout |
| 18 | request_failed |
| 23 | DoorOpenLock |
| 25 | RequestRefused |
| 26 | RequestFailed |

## 7 Section 5 — Live Telemetry Cards

| Card | Signal Source | Unit | Color | Update Source |
| --- | --- | --- | --- | --- |
| Fuel Level | CAN identifier 854 | % | ■ Amber | Vehicle Telemetry tab publish |
| Engine Speed | CAN RPM | RPM | ■ Emerald | Vehicle Telemetry tab / Auto Drive |
| Water Temp | CAN temp | °C | ■ Rose | Vehicle Telemetry tab / Auto Drive |
| Battery | CAN voltage | V | ■ Sky | Vehicle Telemetry tab |
| Odometer | CAN distance | km | ■ Violet | Vehicle Telemetry tab / Auto Drive |
| GPS Lat | Location payload | ° | ■ Teal | Location Telemetry tab / Auto Drive |

Per-card features: animated number count; trend arrow ↑/↓/→; dynamic box-shadow glow when value updates.

## 8 Section 6 — Message Log Panel

| Feature | Detail |
| --- | --- |
| Direction badges | ↑ PUB and ↓ SUB |
| Status badges | SUCCESS FAILED PENDING |
| Max entries | 200 (older entries evicted from state) |
| Payload expansion | Click entry → JSON pretty-print + Hex payload (proto mock, first 96 chars) |
| Filter: Direction | ALL / PUB / SUB toggle buttons |
| Filter: Message type | Free-text search input |
| Filter: Status | ALL / SUCCESS / FAILED / PENDING toggle buttons |
| Clear Log | Animated scatter — entries fly off screen before clearing |
| Export JSON | Downloads all entries as mqtt_log.json via Blob URL |

## 9 Automation — Simulate Auto Drive

Trigger: "⭐ Simulate Auto Drive" toggle button (left column). Must be CONNECTED first.

| Published Every 3s | Behavior |
| --- | --- |
| Location Telemetry | gpsLat/Long drifts by ±0.001° random walk per tick |
| Vehicle Telemetry — Speed | Fluctuates 0–140 km/h with sinusoidal drift |
| Vehicle Telemetry — Fuel | Slowly decreases 0.01% per tick |
| Vehicle Telemetry — RPM | Proportional to speed with noise |
| Vehicle Telemetry — Water Temp | Warms up to ~90°C and stabilizes |

## 10 Complete MQTT Topic Reference

Pattern: /dongle/{VIN}/MQTTPROTOBUF/{type} — Spec §8.1

### Publish Topics — Dongle → CVIP

| # | Topic Suffix | SB API ID | Tab/Source | Implemented |
| --- | --- | --- | --- | --- |
| 1 | deviceJoined | SB-DONGLE_ACTIVATION-007 | Device Join tab | ✅ |
| 2 | vehicleTelemetry | §12.2 | Vehicle Tel tab | ✅ |
| 3 | locationTelemetry | §12.3 | Location tab | ✅ |
| 4 | events | §13.2 | Events tab | ✅ |
| 5 | alerts | §13.1 | Alerts tab | ✅ |
| 6 | tripStart | SB-TRIP_TELEMETRY-006 | Trips → Start | ✅ |
| 7 | tripCurrent | SB-TRIP_TELEMETRY-006 | Trips → Current | ✅ |
| 8 | tripEnd | SB-TRIP_TELEMETRY-006 | Trips → End | ✅ |
| 9 | commandResponse | SB-DONGLE_COMMANDS-001 | Command Inbox | ✅ |
| 10 | fotaCommandResponse | SB-FOTA_COMMANDS-001 | Command Inbox (FOTA) | ✅ |

### Subscribe Topics — CVIP → Dongle (Simulated)

| # | Topic Suffix | SB API ID | Simulated Via | Implemented |
| --- | --- | --- | --- | --- |
| 1 | deviceJoinedRsp | SB-DONGLE_ACTIVATION-007 | Auto-fires 1.2s after deviceJoined publish | ✅ |
| 2 | command | SB-DONGLE_COMMANDS-001 | "↓ Simulate" buttons in Command Inbox | ✅ |
| 3 | fotaCommand | SB-FOTA_COMMANDS-001 | "↓ FOTA Command" button | ✅ |

## 11 State Management Architecture

Global state is managed via React useReducer with a single normalized state tree consisting of 8 top-level slices:

const initialState = {   config:           { vin, imei, tboxSerial, clientId, messageId, correlationId, broker, protocolVersion },   connection:       { status, publishCount, receiveCount, uptimeSeconds, lastPublishAt, connectedAt },   telemetry:        { fuelLevel, engineSpeed, engineWaterTemp, batteryVoltage, odometer, gpsLat, gpsLong ... },   telemetryHistory: { fuelLevel: [...10], engineSpeed: [...10], ... },    // trend arrow computation   log:              [ { id, timestamp, direction, messageType, topic, status, payloadBytes, ... } ],   commandInbox:     [ { id, commandType, correlationId, receivedAt, responseStatus } ],   toasts:           [ { id, message, type } ],   activePublishTab: 'vehicleTelemetry',   autoDriveActive:  false };

#### Reducer Actions

| Action Type | Effect |
| --- | --- |
| SET_CONFIG | Merges new config values from Device Identity panel |
| SET_CONNECTION | Partial merge of connection state slice |
| INCREMENT_PUBLISH | publishCount++, updates lastPublishAt |
| INCREMENT_RECEIVE | receiveCount++, updates lastReceiveAt |
| SET_TELEMETRY | Updates single telemetry key + appends to history array |
| SET_TELEMETRY_BULK | Updates multiple telemetry keys at once |
| ADD_LOG | Prepends entry to log[], max 200 enforced |
| CLEAR_LOG | Resets log to empty array |
| ADD_COMMAND | Appends command to commandInbox[] |
| UPDATE_COMMAND | Updates responseStatus for a specific command |
| ADD_TOAST | Appends toast to toasts[], max 5 |
| REMOVE_TOAST | Removes toast by id (auto after 3500ms) |
| SET_ACTIVE_TAB | Updates activePublishTab |
| SET_AUTO_DRIVE | Toggles autoDriveActive boolean |

## 12 Animation Spring Configurations (Framer Motion)

##### Log Entry Slide-in

stiffness: 300, damping: 28

New log entry animates from bottom

##### Telemetry Number

duration: 0.6s, ease curve

animate() interpolates old→new value

##### Button Press Pop

stiffness: 500, damping: 30

whileTap scale 0.95 on all buttons

##### Toast Notification

stiffness: 400, damping: 20

x: 120→0, scale: 0.85→1

##### Command Inbox Slide

stiffness: 260, damping: 26

Command arrives from right edge

##### GPS Pin Movement

stiffness: 120, damping: 18

useSpring tracks lat/long input changes

##### CONNECTED Dot Pulse

duration: 2.0s, loop infinite

scale [1, 1.5, 1], opacity [1, 0.5, 1]

##### ERROR Shake

stiffness: 800, damping: 10

High stiffness rapid oscillation

##### Log Scatter (Clear)

stiffness: 100, damping: 8

Entries fly off screen before clearing

##### Panel Collapse

stiffness: 350, damping: 30

Device Identity accordion height: 0↔auto

##### Card Glow (Telemetry)

stiffness: 150, damping: 25

Box-shadow intensity on value update

##### Trend Arrow Enter

stiffness: 180, damping: 22

y: -4→0, opacity: 0→1 on value change

## 13 Visual Design System

### 13.1 Color Palette

| Token | Hex Value | Tailwind Equivalent | Usage |
| --- | --- | --- | --- |
| T.bgPrimary | #0d0f12 | — | Page background |
| T.bgCard | #141720 | — | All card backgrounds |
| T.border | #2a2d35 | zinc-800 | All borders, dividers |
| T.accent | #f59e0b | amber-500 | Primary actions, section headers, active tabs |
| T.pub | #10b981 | emerald-500 | Published messages, CONNECTED state |
| T.sub | #38bdf8 | sky-400 | Subscribed messages, broker label text |
| T.error | #f43f5e | rose-500 | Errors, alerts, DISCONNECTED button |
| T.textPri | #e4e4e7 | zinc-200 | All body text and values |
| T.textMut | #52525b | zinc-600 | Labels, timestamps, secondary info |
| T.mono | — | — | 'JetBrains Mono', 'Fira Code', monospace |

### 13.2 Three-Column Layout

┌────────────────────────────────────────────────────────────────────────────────────┐ │ ⚡ MQTT DEVICE  │  VIN: T123ZTZT867657777  │  IMEI: 356741...  │  BROKER  │ STATUS │  ← 46px Status Bar ├────────────────────┬─────────────────────────────────────────┬───────────────────────┤ │   LEFT  (300px)    │         CENTER  (flex-1, min-width:0)   │   RIGHT   (350px)     │ │                    │                                         │                       │ │  ⚙ Device Identity │  ↑ PUBLISH MQTT MESSAGES                │  ⊙ LIVE TELEMETRY     │ │  ┌──────────────┐  │  ┌─────┬──────┬──────┬──────┬──────┬─┐ │  ┌─────────┬────────┐ │ │  │ VIN          │  │  │VTel │ Loc  │Events│Alert │Trip  │D│ │  │Fuel 64% │RPM2800 │ │ │  │ IMEI         │  │  └─────┴──────┴──────┴──────┴──────┴─┘ │  │Temp 90° │Bat12.6 │ │ │  │ TBOX Serial  │  │  [Tab content area — forms + buttons]   │  │Odo45230 │Lat13.08│ │ │  │ Client ID    │  │                                         │  └─────────┴────────┘ │ │  │ Message ID   │  │  ↓ COMMAND INBOX                        │                       │ │  │ Corr. ID     │  │  [↓ DoorLock] [↓ DoorUnlock]           │  ≡ MESSAGE LOG (n)     │ │  │ Broker URL   │  │  [↓ BlinkerON][↓ BlinkerOFF]           │  [filters][clear][exp] │ │  │ Protocol Ver │  │  [↓ Honk    ] [↓ FOTA      ]           │  [entries...]          │ │  └──────────────┘  │  [↓ OTA     ] [↓ FetchLog  ]           │                       │ │  ◎ MQTT CONNECTION │  [inbox entries with correlationId]     │                       │ │  ⭐ Automation      │                                         │                       │ └────────────────────┴─────────────────────────────────────────┴───────────────────────┘

## 14 What Is NOT Implemented (Scope Limitations)

| Feature | Reason / Note | Spec Reference |
| --- | --- | --- |
| ✗ Real MQTT TLS connection | No real broker needed — all simulated in-memory | §6.2, §6.2.6 |
| ✗ Real protobuf binary encoding | Browser security; protobufjs not bundled; mock hex used instead | All proto files |
| ✗ TLS certificates (ca.crt, client.crt, client.key) | Browser Web Crypto API restrictions on raw TLS socket | §6.2.6 |
| ✗ LWT (Last Will and Testament) | Server-side MQTT broker config — not a dashboard UI concern | §6.2.5 |
| ✗ Exponential retry logic (7 steps, 2s→128s) | Simulated with flat 800ms fake latency | §7.3.2 |
| ✗ Buffered /history topics | /vehicleTelemetry/history etc. not exposed in UI | §8.2 |
| ✗ remoteSMSWakeup flow | Not included in source scripts scope | §10.13 |
| ✗ Diagnostic Alerts topic | Removed from spec in §3.3 update (21-Jan-26) and §4.1 | §13.3 (removed) |
| ✗ Emergency & Safety Alerts | Separate emergencyAlerts topic not in source scripts | §13.4 |
| ✗ FOTA HTTPS download | Binary firmware download is server-initiated, not device UI | §11.2 |
| ✗ userDefinedSpeedCommand | Removed from spec in §4.1 update (06-Feb-26) | §9.5 (removed) |
| ✗ IMEI Upload / Supplier Feed APIs | REST API flows (not MQTT); backend-only | §7.2.1, §7.2.3 |

## 15 How to Use the Dashboard

### Quick Start Workflow

| Step | Action | Expected Result |
| --- | --- | --- |
| 1 | Navigate to /mqtt-virtual-device in Rule Engine | Dashboard loads with dark theme, all 3 columns visible |
| 2 | Review VIN/IMEI in Device Identity panel (left col) | Fields pre-filled from .env defaults |
| 3 | Click ↻ Generate UUIDs | messageId and correlationId refresh to new UUIDs |
| 4 | Click 💾 Save Config | Green toast "✓ Config saved" appears top-right |
| 5 | Click ▶ CONNECT | Status → CONNECTING (amber pulse) → CONNECTED (green pulse) in ~800ms |
| 6 | Select Vehicle Tel tab → choose signal (e.g., Fuel Level) → click ↑ Publish | Log entry appears in Message Log; Fuel Level card updates |
| 7 | Select Location tab → change lat/long → click ↑ Publish Location | GPS pin moves on mini-map; GPS Lat card updates |
| 8 | Select Device Join tab → click ↑ Publish deviceJoined | deviceJoined published; 1.2s later a SUB entry for deviceJoinedRsp appears in log |
| 9 | In Command Inbox, click ↓ Remote Door Lock | Command slides in with correlationId; click ✓ to send commandResponse |
| 10 | Click ⭐ Simulate Auto Drive | GPS and telemetry update every 3s; trend arrows activate |
| 11 | Click Export JSON in Message Log | Downloads mqtt_log.json with all entries |

✅ Publishing Pre-requisite

The dashboard enforces that

you must be CONNECTED before any publish is accepted.

Attempting to publish while DISCONNECTED silently ignores the action (no log entry created).   This matches real MQTT behavior.

Generated: 15-Apr-2026  ·  Rule Engine Dashboard v2.0  ·  Component: MQTTVirtualDeviceDashboard.jsx
   References: MQTT_Virtual_Device_Rev1.1 (15-Apr-2026) + TE 01 STLA M6 SB Interface Spec V5.0 (17-Feb-2026)
   Prepared by: Antigravity AI / Tata Elxsi — Confidential